Power system for PH-ANR based on Jabiru 3300 engine and AeroElectric Z-21A.

Main design considerations:

- Battery contactor should cut ALL power including stuck starter.

- Shortest path for high-current starter

- Overvoltage protection against Alternator and regulator failure.

- Alternator cannot be engaged without battery enabled.

- Warning light should show when power comes from battery only.

- Current measured by Westach V/A meter shows load and charge but not starter current.

- Large capacitor and discharge resistor across regulator avoids peaks.

Wire sizes:

- Battery, main contactor, starter contactor, starter:	AWG #4

- Shunt, main bus, alternator, regulator circuit:		AWG #12

- Switches, indicators:						AWG #18